NAV2 FAILURE OBSERVATIONS:|

IN ROBOT:

1. ros2 launch my-ros-bot diffbot.launch.py use_mock_hardware:=false gui:=false

[diffbot launch in robot;  launches ros2_control_node, robot_state_publisher,  ekf_filter_node, joint_state_broadcaster, diffbot_base_controller, bno055_driver_node ]

if NORMAL:

[ros2_control_node-1] [INFO] [1779346213.088757771] [controller_manager.resource_manager.hardware_component.system.DiffBot]: CMD: FL=0.00 FR=0.00 RL=0.00 RR=0.00

[ros2_control_node-1] [INFO] [1779346214.108655682] [controller_manager.resource_manager.hardware_component.system.DiffBot]: CMD: FL=0.00 FR=0.00 RL=0.00 RR=0.00

Sometimes:

[ros2_control_node-1] [INFO] [1779346197.948573309] [controller_manager.resource_manager.hardware_component.system.DiffBot]: CMD: FL=0.00 FR=0.00 RL=0.00 RR=0.00

[ros2_control_node-1] [INFO] [1779346198.968612604] [controller_manager.resource_manager.hardware_component.system.DiffBot]: CMD: FL=0.00 FR=0.00

2. ros2 launch sllidar_ros2 sllidar_s2_launch.py

[INFO] [launch]: Default logging verbosity is set to INFO

[INFO] [sllidar_node-1]: process started with pid [173]

[sllidar_node-1] [INFO] [1779345824.772801623] [sllidar_node]: SLLidar running on ROS2 package SLLidar.ROS2 SDK Version:1.0.1, SLLIDAR SDK Version:2.1.0

[sllidar_node-1] [INFO] [1779345824.791235338] [sllidar_node]: SLLidar S/N: 685EE18BC7E49CCDA3EB9AF4FC834C73

[sllidar_node-1] [INFO] [1779345824.791468504] [sllidar_node]: Firmware Ver: 1.02

[sllidar_node-1] [INFO] [1779345824.791520268] [sllidar_node]: Hardware Rev: 18

[sllidar_node-1] [INFO] [1779345824.796551852] [sllidar_node]: SLLidar health status : 0

[sllidar_node-1] [INFO] [1779345824.796902475] [sllidar_node]: SLLidar health status : OK.

[sllidar_node-1] [INFO] [1779345825.032661547] [sllidar_node]: current scan mode: DenseBoost, sample rate: 32 Khz, max_distance: 18.0 m, scan frequency:5.0 Hz,

IN HOST:

1. Rviz2

2. ros2 launch nav2_bringup localization_launch.py   map:=lab.yaml use_sim_time:=false   map_subscribe_transient_local:=true   params_file:=src/my-ros-bot/config/nav2_params.yaml

[INFO] [launch]: All log files can be found below /home/anandhu-sudha/.ros/log/2026-05-21-12-14-20-691273-anandhu-12348

[INFO] [launch]: Default logging verbosity is set to INFO

[INFO] [map_server-1]: process started with pid [12349]

[INFO] [amcl-2]: process started with pid [12351]

[INFO] [lifecycle_manager-3]: process started with pid [12353]

[lifecycle_manager-3] [INFO] [1779345860.874781138] [lifecycle_manager_localization]: Creating

[lifecycle_manager-3] [INFO] [1779345860.876987359] [lifecycle_manager_localization]: Creating and initializing lifecycle service clients

[lifecycle_manager-3] [INFO] [1779345860.886391465] [lifecycle_manager_localization]: Starting managed nodes bringup...

[lifecycle_manager-3] [INFO] [1779345860.886427766] [lifecycle_manager_localization]: Configuring map_server

[amcl-2] [INFO] [1779345860.886391396] [amcl]:

[amcl-2] 	amcl lifecycle node launched.

[amcl-2] 	Waiting on external lifecycle transitions to activate

[amcl-2] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[map_server-1] [INFO] [1779345860.886577328] [map_server]:

[map_server-1] 	map_server lifecycle node launched.

[map_server-1] 	Waiting on external lifecycle transitions to activate

[map_server-1] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[map_server-1] [INFO] [1779345860.886666208] [map_server]: Creating

[amcl-2] [INFO] [1779345860.886710199] [amcl]: Creating

[map_server-1] [INFO] [1779345860.886954057] [map_server]: Configuring

[map_server-1] [INFO] [1779345860.886989494] [map_io]: Loading yaml file: lab.yaml

[map_server-1] [INFO] [1779345860.887226433] [map_io]: resolution: 0.05

[map_server-1] [INFO] [1779345860.887233918] [map_io]: origin[0]: -2.28

[map_server-1] [INFO] [1779345860.887238094] [map_io]: origin[1]: -2.31

[map_server-1] [INFO] [1779345860.887242955] [map_io]: origin[2]: 0

[map_server-1] [INFO] [1779345860.887248196] [map_io]: free_thresh: 0.25

[map_server-1] [INFO] [1779345860.887253615] [map_io]: occupied_thresh: 0.65

[map_server-1] [INFO] [1779345860.887259360] [map_io]: mode: trinary

[map_server-1] [INFO] [1779345860.887263585] [map_io]: negate: 0

[map_server-1] [INFO] [1779345860.887411178] [map_io]: Loading image_file: ./lab.pgm

[map_server-1] [INFO] [1779345860.888295600] [map_io]: Read map ./lab.pgm: 119 X 73 map @ 0.05 m/cell

[lifecycle_manager-3] [INFO] [1779345860.892402484] [lifecycle_manager_localization]: Configuring amcl

[amcl-2] [INFO] [1779345861.023556083] [amcl]: Configuring

[amcl-2] [INFO] [1779345861.023702750] [amcl]: initTransforms

[amcl-2] [INFO] [1779345861.028822133] [amcl]: initPubSub

[amcl-2] [INFO] [1779345861.031134372] [amcl]: Subscribed to map topic.

[lifecycle_manager-3] [INFO] [1779345861.032605485] [lifecycle_manager_localization]: Activating map_server

[map_server-1] [INFO] [1779345861.032705988] [map_server]: Activating

[map_server-1] [INFO] [1779345861.032804493] [map_server]: Creating bond (map_server) to lifecycle manager.

[amcl-2] [INFO] [1779345861.032864086] [amcl]: Received a 119 X 73 map @ 0.050 m/pix

[lifecycle_manager-3] [INFO] [1779345861.135548737] [lifecycle_manager_localization]: Server map_server connected with bond.

[lifecycle_manager-3] [INFO] [1779345861.135583715] [lifecycle_manager_localization]: Activating amcl

[amcl-2] [INFO] [1779345861.135739796] [amcl]: Activating

[amcl-2] [INFO] [1779345861.135775989] [amcl]: Creating bond (amcl) to lifecycle manager.

[lifecycle_manager-3] [INFO] [1779345861.237631645] [lifecycle_manager_localization]: Server amcl connected with bond.

[lifecycle_manager-3] [INFO] [1779345861.237666858] [lifecycle_manager_localization]: Managed nodes are active

[lifecycle_manager-3] [INFO] [1779345861.237677773] [lifecycle_manager_localization]: Creating bond timer...

[amcl-2] [INFO] [1779345865.967263176] [amcl]: createLaserObject

[amcl-2] [WARN] [1779345865.974181983] [amcl]: AMCL cannot publish a pose or update the transform. Please set the initial pose...

[amcl-2] [WARN] [1779345868.016113272] [amcl]: AMCL cannot publish a pose or update the transform. Please set the initial pose...

[amcl-2] [WARN] [1779345870.108907872] [amcl]: AMCL cannot publish a pose or update the transform. Please set the initial pose...

[amcl-2] [WARN] [1779345872.109024580] [amcl]: AMCL cannot publish a pose or update the transform. Please set the initial pose...

[amcl-2] [INFO] [1779345872.706203297] [amcl]: initialPoseReceived

[amcl-2] [WARN] [1779345872.706291543] [amcl]: Failed to transform initial pose in time (Lookup would require extrapolation into the future.  Requested time 1779345872.706237 but the latest data is at time 1779345872.607645, when looking up transform from frame [base_link] to frame [odom])

3. ros2 launch nav2_bringup navigation_launch.py   use_sim_time:=false   params_file:=src/my-ros-bot/config/nav2_params.yaml

[INFO] [launch]: All log files can be found below /home/anandhu-sudha/.ros/log/2026-05-21-12-19-41-699512-anandhu-12518

[INFO] [launch]: Default logging verbosity is set to INFO

[INFO] [controller_server-1]: process started with pid [12520]

[INFO] [smoother_server-2]: process started with pid [12522]

[INFO] [planner_server-3]: process started with pid [12524]

[INFO] [behavior_server-4]: process started with pid [12526]

[INFO] [bt_navigator-5]: process started with pid [12528]

[INFO] [waypoint_follower-6]: process started with pid [12530]

[INFO] [velocity_smoother-7]: process started with pid [12532]

[INFO] [lifecycle_manager-8]: process started with pid [12534]

[lifecycle_manager-8] [INFO] [1779346182.250754346] [lifecycle_manager_navigation]: Creating

[bt_navigator-5] [INFO] [1779346182.251991177] [bt_navigator]:

[bt_navigator-5] 	bt_navigator lifecycle node launched.

[bt_navigator-5] 	Waiting on external lifecycle transitions to activate

[bt_navigator-5] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[bt_navigator-5] [INFO] [1779346182.252147193] [bt_navigator]: Creating

[lifecycle_manager-8] [INFO] [1779346182.263241040] [lifecycle_manager_navigation]: Creating and initializing lifecycle service clients

[waypoint_follower-6] [INFO] [1779346182.279233183] [waypoint_follower]:

[waypoint_follower-6] 	waypoint_follower lifecycle node launched.

[waypoint_follower-6] 	Waiting on external lifecycle transitions to activate

[waypoint_follower-6] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[behavior_server-4] [INFO] [1779346182.279233237] [behavior_server]:

[behavior_server-4] 	behavior_server lifecycle node launched.

[behavior_server-4] 	Waiting on external lifecycle transitions to activate

[behavior_server-4] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[waypoint_follower-6] [INFO] [1779346182.279681462] [waypoint_follower]: Creating

[smoother_server-2] [INFO] [1779346182.285195552] [smoother_server]:

[smoother_server-2] 	smoother_server lifecycle node launched.

[smoother_server-2] 	Waiting on external lifecycle transitions to activate

[smoother_server-2] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[smoother_server-2] [INFO] [1779346182.286805755] [smoother_server]: Creating smoother server

[controller_server-1] [INFO] [1779346182.288033194] [controller_server]:

[controller_server-1] 	controller_server lifecycle node launched.

[controller_server-1] 	Waiting on external lifecycle transitions to activate

[controller_server-1] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[velocity_smoother-7] [INFO] [1779346182.291456249] [velocity_smoother]:

[velocity_smoother-7] 	velocity_smoother lifecycle node launched.

[velocity_smoother-7] 	Waiting on external lifecycle transitions to activate

[velocity_smoother-7] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[controller_server-1] [INFO] [1779346182.291616294] [controller_server]: Creating controller server

[planner_server-3] [INFO] [1779346182.292550448] [planner_server]:

[planner_server-3] 	planner_server lifecycle node launched.

[planner_server-3] 	Waiting on external lifecycle transitions to activate

[planner_server-3] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[planner_server-3] [INFO] [1779346182.293595006] [planner_server]: Creating

[controller_server-1] [INFO] [1779346182.304524630] [local_costmap.local_costmap]:

[controller_server-1] 	local_costmap lifecycle node launched.

[controller_server-1] 	Waiting on external lifecycle transitions to activate

[controller_server-1] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[controller_server-1] [INFO] [1779346182.305778514] [local_costmap.local_costmap]: Creating Costmap

[planner_server-3] [INFO] [1779346182.306381456] [global_costmap.global_costmap]:

[planner_server-3] 	global_costmap lifecycle node launched.

[planner_server-3] 	Waiting on external lifecycle transitions to activate

[planner_server-3] 	See https://design.ros2.org/articles/node_lifecycle.html for more information.

[planner_server-3] [INFO] [1779346182.306828306] [global_costmap.global_costmap]: Creating Costmap

[lifecycle_manager-8] [INFO] [1779346182.928701933] [lifecycle_manager_navigation]: Starting managed nodes bringup...

[lifecycle_manager-8] [INFO] [1779346182.928751322] [lifecycle_manager_navigation]: Configuring controller_server

[controller_server-1] [INFO] [1779346182.929016170] [controller_server]: Configuring controller interface

[controller_server-1] [INFO] [1779346182.929273424] [controller_server]: getting goal checker plugins..

[controller_server-1] [INFO] [1779346182.929356347] [controller_server]: Controller frequency set to 20.0000Hz

[controller_server-1] [INFO] [1779346182.929401791] [local_costmap.local_costmap]: Configuring

[controller_server-1] [INFO] [1779346182.933576787] [local_costmap.local_costmap]: Using plugin "voxel_layer"

[controller_server-1] [INFO] [1779346182.936059829] [local_costmap.local_costmap]: Subscribed to Topics: scan

[controller_server-1] [INFO] [1779346182.940910084] [local_costmap.local_costmap]: Initialized plugin "voxel_layer"

[controller_server-1] [INFO] [1779346182.940946172] [local_costmap.local_costmap]: Using plugin "inflation_layer"

[controller_server-1] [INFO] [1779346182.941380539] [local_costmap.local_costmap]: Initialized plugin "inflation_layer"

[controller_server-1] [INFO] [1779346182.948270975] [controller_server]: Created progress_checker : progress_checker of type nav2_controller::SimpleProgressChecker

[controller_server-1] [INFO] [1779346182.948809892] [controller_server]: Created goal checker : general_goal_checker of type nav2_controller::SimpleGoalChecker

[controller_server-1] [INFO] [1779346182.949113239] [controller_server]: Controller Server has general_goal_checker  goal checkers available.

[controller_server-1] [INFO] [1779346182.950788813] [controller_server]: Created controller : FollowPath of type dwb_core::DWBLocalPlanner

[controller_server-1] [INFO] [1779346182.951283855] [controller_server]: Setting transform_tolerance to 1.000000

[controller_server-1] [INFO] [1779346182.960653630] [controller_server]: Using critic "RotateToGoal" (dwb_critics::RotateToGoalCritic)

[controller_server-1] [INFO] [1779346182.960966034] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.961052205] [controller_server]: Using critic "Oscillation" (dwb_critics::OscillationCritic)

[controller_server-1] [INFO] [1779346182.961355383] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.961431624] [controller_server]: Using critic "BaseObstacle" (dwb_critics::BaseObstacleCritic)

[controller_server-1] [INFO] [1779346182.961536430] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.961606669] [controller_server]: Using critic "GoalAlign" (dwb_critics::GoalAlignCritic)

[controller_server-1] [INFO] [1779346182.961814114] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.961899551] [controller_server]: Using critic "PathAlign" (dwb_critics::PathAlignCritic)

[controller_server-1] [INFO] [1779346182.962467522] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.962618686] [controller_server]: Using critic "PathDist" (dwb_critics::PathDistCritic)

[controller_server-1] [INFO] [1779346182.962821757] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.962915708] [controller_server]: Using critic "GoalDist" (dwb_critics::GoalDistCritic)

[controller_server-1] [INFO] [1779346182.963073514] [controller_server]: Critic plugin initialized

[controller_server-1] [INFO] [1779346182.963096215] [controller_server]: Controller Server has FollowPath  controllers available.

[lifecycle_manager-8] [INFO] [1779346182.968320351] [lifecycle_manager_navigation]: Configuring smoother_server

[smoother_server-2] [INFO] [1779346182.968479357] [smoother_server]: Configuring smoother server

[smoother_server-2] [INFO] [1779346182.974035749] [smoother_server]: Created smoother : simple_smoother of type nav2_smoother::SimpleSmoother

[smoother_server-2] [INFO] [1779346182.974552144] [smoother_server]: Smoother Server has simple_smoother  smoothers available.

[lifecycle_manager-8] [INFO] [1779346182.978493092] [lifecycle_manager_navigation]: Configuring planner_server

[planner_server-3] [INFO] [1779346182.978662902] [planner_server]: Configuring

[planner_server-3] [INFO] [1779346182.978686196] [global_costmap.global_costmap]: Configuring

[planner_server-3] [INFO] [1779346182.982235548] [global_costmap.global_costmap]: Using plugin "static_layer"

[planner_server-3] [INFO] [1779346182.984724059] [global_costmap.global_costmap]: Subscribing to the map topic (/map) with transient local durability

[planner_server-3] [INFO] [1779346182.985957275] [global_costmap.global_costmap]: Initialized plugin "static_layer"

[planner_server-3] [INFO] [1779346182.985991415] [global_costmap.global_costmap]: Using plugin "obstacle_layer"

[planner_server-3] [INFO] [1779346182.986446707] [global_costmap.global_costmap]: Subscribed to Topics: scan

[planner_server-3] [INFO] [1779346182.989203638] [global_costmap.global_costmap]: Initialized plugin "obstacle_layer"

[planner_server-3] [INFO] [1779346182.989229878] [global_costmap.global_costmap]: Using plugin "inflation_layer"

[planner_server-3] [INFO] [1779346182.989620269] [global_costmap.global_costmap]: Initialized plugin "inflation_layer"

[planner_server-3] [ERROR] [1779346182.997540881] [global_costmap.global_costmap]: The configured inflation radius (0.200) is smaller than the computed inscribed radius (0.225) of your footprint, it is highly recommended to set inflation radius to be at least as big as the inscribed radius to avoid collisions

[planner_server-3] [INFO] [1779346183.000720697] [planner_server]: Created global planner plugin GridBased of type nav2_navfn_planner/NavfnPlanner

[planner_server-3] [INFO] [1779346183.000753948] [planner_server]: Configuring plugin GridBased of type NavfnPlanner

[planner_server-3] [INFO] [1779346183.001062447] [planner_server]: Planner Server has GridBased  planners available.

[lifecycle_manager-8] [INFO] [1779346183.007363769] [lifecycle_manager_navigation]: Configuring behavior_server

[behavior_server-4] [INFO] [1779346183.007611058] [behavior_server]: Configuring

[behavior_server-4] [INFO] [1779346183.014816878] [behavior_server]: Creating behavior plugin spin of type nav2_behaviors/Spin

[behavior_server-4] [INFO] [1779346183.015668675] [behavior_server]: Configuring spin

[behavior_server-4] [INFO] [1779346183.022169414] [behavior_server]: Creating behavior plugin backup of type nav2_behaviors/BackUp

[behavior_server-4] [INFO] [1779346183.022725044] [behavior_server]: Configuring backup

[behavior_server-4] [INFO] [1779346183.027759720] [behavior_server]: Creating behavior plugin drive_on_heading of type nav2_behaviors/DriveOnHeading

[behavior_server-4] [INFO] [1779346183.028377070] [behavior_server]: Configuring drive_on_heading

[behavior_server-4] [INFO] [1779346183.033671358] [behavior_server]: Creating behavior plugin assisted_teleop of type nav2_behaviors/AssistedTeleop

[behavior_server-4] [INFO] [1779346183.035531498] [behavior_server]: Configuring assisted_teleop

[behavior_server-4] [INFO] [1779346183.065720828] [behavior_server]: Creating behavior plugin wait of type nav2_behaviors/Wait

[behavior_server-4] [INFO] [1779346183.066255262] [behavior_server]: Configuring wait

[lifecycle_manager-8] [INFO] [1779346183.069691535] [lifecycle_manager_navigation]: Configuring bt_navigator

[bt_navigator-5] [INFO] [1779346183.069880636] [bt_navigator]: Configuring

[lifecycle_manager-8] [INFO] [1779346183.126789524] [lifecycle_manager_navigation]: Configuring waypoint_follower

[waypoint_follower-6] [INFO] [1779346183.126956179] [waypoint_follower]: Configuring

[waypoint_follower-6] [INFO] [1779346183.135895265] [waypoint_follower]: Created waypoint_task_executor : wait_at_waypoint of type nav2_waypoint_follower::WaitAtWaypoint

[lifecycle_manager-8] [INFO] [1779346183.137042149] [lifecycle_manager_navigation]: Configuring velocity_smoother

[velocity_smoother-7] [INFO] [1779346183.137313411] [velocity_smoother]: Configuring velocity smoother

[lifecycle_manager-8] [INFO] [1779346183.140629644] [lifecycle_manager_navigation]: Activating controller_server

[controller_server-1] [INFO] [1779346183.140835793] [controller_server]: Activating

[controller_server-1] [INFO] [1779346183.140877580] [local_costmap.local_costmap]: Activating

[controller_server-1] [INFO] [1779346183.140895226] [local_costmap.local_costmap]: Checking transform

[controller_server-1] [INFO] [1779346183.140925202] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[planner_server-3] [INFO] [1779346183.318482036] [global_costmap.global_costmap]: StaticLayer: Resizing costmap to 119 X 73 at 0.050000 m/pix

[controller_server-1] [INFO] [1779346183.641045741] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346184.140973869] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346184.641005870] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346185.140972146] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346185.641040863] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346186.140989722] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346186.641008194] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346187.140976520] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346187.640978371] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346188.141000143] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346188.641009042] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346189.140994716] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346189.641027422] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346190.140974487] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346190.641019718] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346191.140985836] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346191.641012492] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346192.140992539] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346192.641008341] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346193.140971764] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346193.640997539] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346194.141002053] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346194.641048558] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346195.140972477] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346195.641164023] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346196.141001989] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346196.640973947] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346197.141018246] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346197.641010003] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346198.140969946] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346198.640967873] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346199.140983929] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346199.640968539] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346200.140937041] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346200.640969585] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346201.140984119] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346201.640976013] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346202.141036333] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346202.640971257] [local_costmap.local_costmap]: Timed out waiting for transform from base_link to odom to become available, tf error: Invalid frame ID "base_link" passed to canTransform argument source_frame - frame does not exist

[controller_server-1] [INFO] [1779346203.141091754] [local_costmap.local_costmap]: start

[controller_server-1] [INFO] [1779346203.342485626] [controller_server]: Creating bond (controller_server) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346203.447471285] [lifecycle_manager_navigation]: Server controller_server connected with bond.

[lifecycle_manager-8] [INFO] [1779346203.447534444] [lifecycle_manager_navigation]: Activating smoother_server

[smoother_server-2] [INFO] [1779346203.447968670] [smoother_server]: Activating

[smoother_server-2] [INFO] [1779346203.448040978] [smoother_server]: Creating bond (smoother_server) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346203.551994678] [lifecycle_manager_navigation]: Server smoother_server connected with bond.

[lifecycle_manager-8] [INFO] [1779346203.552030595] [lifecycle_manager_navigation]: Activating planner_server

[planner_server-3] [INFO] [1779346203.552407284] [planner_server]: Activating

[planner_server-3] [INFO] [1779346203.552461375] [global_costmap.global_costmap]: Activating

[planner_server-3] [INFO] [1779346203.552471822] [global_costmap.global_costmap]: Checking transform

[planner_server-3] [INFO] [1779346203.552637695] [global_costmap.global_costmap]: start

[planner_server-3] [INFO] [1779346204.603865951] [planner_server]: Activating plugin GridBased of type NavfnPlanner

[planner_server-3] [INFO] [1779346204.605036428] [planner_server]: Creating bond (planner_server) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346204.768573791] [lifecycle_manager_navigation]: Server planner_server connected with bond.

[lifecycle_manager-8] [INFO] [1779346204.768636226] [lifecycle_manager_navigation]: Activating behavior_server

[behavior_server-4] [INFO] [1779346204.768880955] [behavior_server]: Activating

[behavior_server-4] [INFO] [1779346204.768903145] [behavior_server]: Activating spin

[behavior_server-4] [INFO] [1779346204.768913008] [behavior_server]: Activating backup

[behavior_server-4] [INFO] [1779346204.768919868] [behavior_server]: Activating drive_on_heading

[behavior_server-4] [INFO] [1779346204.768926372] [behavior_server]: Activating assisted_teleop

[behavior_server-4] [INFO] [1779346204.768932512] [behavior_server]: Activating wait

[behavior_server-4] [INFO] [1779346204.768942183] [behavior_server]: Creating bond (behavior_server) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346204.874509851] [lifecycle_manager_navigation]: Server behavior_server connected with bond.

[lifecycle_manager-8] [INFO] [1779346204.874553199] [lifecycle_manager_navigation]: Activating bt_navigator

[bt_navigator-5] [INFO] [1779346204.874990588] [bt_navigator]: Activating

[bt_navigator-5] [INFO] [1779346205.254352830] [bt_navigator]: Creating bond (bt_navigator) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346205.382515317] [lifecycle_manager_navigation]: Server bt_navigator connected with bond.

[lifecycle_manager-8] [INFO] [1779346205.382594397] [lifecycle_manager_navigation]: Activating waypoint_follower

[waypoint_follower-6] [INFO] [1779346205.382984219] [waypoint_follower]: Activating

[waypoint_follower-6] [INFO] [1779346205.383031794] [waypoint_follower]: Creating bond (waypoint_follower) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346205.487788102] [lifecycle_manager_navigation]: Server waypoint_follower connected with bond.

[lifecycle_manager-8] [INFO] [1779346205.487870782] [lifecycle_manager_navigation]: Activating velocity_smoother

[velocity_smoother-7] [INFO] [1779346205.488168061] [velocity_smoother]: Activating

[velocity_smoother-7] [INFO] [1779346205.488233612] [velocity_smoother]: Creating bond (velocity_smoother) to lifecycle manager.

[lifecycle_manager-8] [INFO] [1779346205.592410448] [lifecycle_manager_navigation]: Server velocity_smoother connected with bond.

[lifecycle_manager-8] [INFO] [1779346205.592467870] [lifecycle_manager_navigation]: Managed nodes are active

[lifecycle_manager-8] [INFO] [1779346205.592479212] [lifecycle_manager_navigation]: Creating bond timer...

[bt_navigator-5] [INFO] [1779346586.842020130] [bt_navigator]: Begin navigating from current location (1.98, -0.62) to (0.97, -0.42)

[controller_server-1] [INFO] [1779346586.863294192] [controller_server]: Received a goal, begin computing control effort.  // first goal given

[controller_server-1] [WARN] [1779346586.863366103] [controller_server]: No goal checker was specified in parameter 'current_goal_checker'. Server will use only plugin loaded general_goal_checker . This warning will appear once.

[controller_server-1] [INFO] [1779346587.913547154] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346588.963548836] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346589.963547243] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346591.013547667] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346591.617977345] [controller_server]: Reached the goal! //// first goal reached

[bt_navigator-5] [INFO] [1779346591.652310762] [bt_navigator]: Goal succeeded

[bt_navigator-5] [INFO] [1779346598.799654328] [bt_navigator]: Begin navigating from current location (1.10, -0.55) to (0.28, -0.13)

[controller_server-1] [INFO] [1779346598.820135525] [controller_server]: Received a goal, begin computing control effort. // second goal given

[controller_server-1] [INFO] [1779346599.870342976] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346600.920344084] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346601.920343710] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346602.970343519] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346603.970368393] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346605.020341914] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346606.020345142] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346607.070342744] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346608.120341713] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346609.120344999] [controller_server]: Passing new path to controller.

[controller_server-1] [ERROR] [1779346609.525130928] [DWBLocalPlanner]: No valid trajectories out of 419!   // turned off the robot , was about to hit obstacle

[controller_server-1] [ERROR] [1779346609.525213538] [DWBLocalPlanner]: 0.52: BaseObstacle/Trajectory Hits Obstacle.

[controller_server-1] [ERROR] [1779346609.525226215] [DWBLocalPlanner]: 0.48: Oscillation/Trajectory is oscillating.

[controller_server-1] [WARN] [1779346609.525538536] [controller_server]: No valid trajectories out of 419!

[controller_server-1] [INFO] [1779346610.170419297] [controller_server]: Passing new path to controller.

[controller_server-1] [INFO] [1779346611.170342899] [controller_server]: Passing new path to controller.

[controller_server-1] [ERROR] [1779346611.176003060] [tf_help]: Transform data too old when converting from map to odom

[controller_server-1] [ERROR] [1779346611.176072258] [tf_help]: Data time: 1779346611s 150073573ns, Transform time: 1779346610s 441655910ns